## 19.3	Specific Information Requirements

In addition to the general information required by 19.2 above, where the proposed activity involves the following activities, the information listed will be required.

19.3.3	The taking of surface water or groundwater

- A description of the quantity, rate and timing, including the 7-day take, of the proposed take and an assessment of the need for the take;

- A statement of the intended use for which the water is to be taken;

- A description of the means of the take, delivery, storage (if any) and application to be used;

- With respect to an application for a new take, an assessment of the effect of the take on other users of the source water body;

- In the case of the taking of groundwater, a description of the bore used or to be used and bore head management;

- In the case of the taking of groundwater, a description of the likely adverse effect on the aquifer or any connected surface water body;

- In the case of the taking of groundwater for irrigation purposes, a description of the quality of the groundwater where there is likely to be any adverse effect on soils;

- In the case of any resource consent application for the taking of water under Policy 7.3.3, an assessment of the effects of the activity on:

- The values identified in Schedules 7A, 7B and 7C for any affected water body; and

- Significant indigenous vegetation and significant habitats of indigenous fauna; and

- Instream values; and

- The natural character of any affected water body; and

- The amenity values supported by any affected water body.

19.3.4	The taking of geothermal water

- A description of the volume and of the proposed take and an assessment of the need for the take;

- A statement of the intended use for which the water is to be taken;

- A description of the means of the take, delivery, storage (if any) and application to be used.

- With respect to an application for a new take, an assessment of the effect of the take on other users of the geothermal system and the geothermal system itself;

- In the case of the taking of geothermal water, a description of the likely adverse effect on spring flows and the quality of waterbodies receiving wastewater.

19.3.5	The damming or diversion of water

1.	An assessment of the effects of the activity on:

- The values set out in Schedules 7A, 7B and 7C for any affected water body; and

- Significant indigenous vegetation and significant habitats of indigenous fauna; and

- The natural character of any affected water body; and

- The amenity values supported by any affected water body; and

- Other users of any water or water body affected by the activity; and

- The movement of water and sediment; and

- Any defence against water;

- Adjacent land;

- Fish passage;

- An assessment of the effect on upstream and downstream users of any affected water bodies, land or water, including any likely effect should a dam fail or be overtopped either during or after construction;

- A description of the anticipated effect of the activity on public access to or along the water body including a description of:

- The extent to which members of the public would be excluded or restricted from the area; and

- Where existing public access would be excluded or restricted as a result of the activity, a description of the methods, if any, proposed to bring about enhanced access in the area or elsewhere;

- An assessment of the effect of the activity on any natural hazard, and the extent to which it is likely to create or exacerbate a natural hazard;

- An assessment of the effects of the activity on heritage values in any district plan, any archaeological site, or any place with interim historic place registration including interim registration;

- A description of the provisions made for the remediation of any adverse effect of the failure or overtopping of the dam;

- In the case of a dam, the intended timing and duration of the filling of any reservoir and the proposed discharges from the dam;

- A description of the flow regime intended to be maintained in the water body downstream of the dam or diversion;

- In the case of a diversion, the total quantity or proportion of the flow that is intended to be diverted;

- An assessment of any known contaminated land, for example a recognised “contaminated site” that may be flooded or inundated by the damming or diversion;

- In the case of a flood detention dam, a description of the mechanism for releasing water;

- An assessment of the effects on cultural values.