Addendum notes to the recommended conditions

| Condition(s) | Notes |
| --- | --- |
| 3b. | WCRC compliance staff have recently received training to the detection and reporting of odour. |
| 8. | Karbon King (fly ash) and animal carcases are not included in the approved feedstocks for composting. As a consequence of the removal of Karbon King will make redundant the necessity to monitor the soils for metals. |
| 9. | The permissible quantities have been substantially reduced from the initial application made to the WCRC. |
| 10 & 11. | Reflect recommendations made by Dr. Joe Wildy made during a high-level condition review (see attachment) which are consistent with similar discharge permits for biosolids. |
| 13. | Requirement for a weather station to provide for key compost management operations. |
| 24. | To include an immediate update if substantiated complaints and/or non-compliances are experienced. |
| 27. | Reflects the permitted activity rule for fertiliser discharge within the Regional Land and Water Plan (i.e. Rule 74). |
| 29. | Sampling of the compost in accordance with the New Zealand Standards for Composts, Soil Conditions and Mulches NZS 4454:2005, Table 3.1. |
| 30. | Monitoring includes the compost leachate itself (i.e. Point E) for 12 months from the onset of discharging compost leachate. |
| 32c. | No monitoring when creek is high, a recommendation from the WCRC principal scientist. |
| 33. | New compliance limits for E. coli depending on the upstream sample. |
| 36 – 38. | New conditions for the requirement of a purpose built bunder for the receipt of DAF wastewater due to its potential for odour emissions. |
| Term. | Reduced from the amended application of 15 years (35 years in the original) to 10 years. |

# APPENDIX 1 – RECOMMENDED CONDITIONS

## PURPOSE OF RESOURCE CONSENT

| ## Consent No | ## Consent Type | ## Activity / Location |
| --- | --- | --- |
| RC-2024-0136-01 | Discharge Permit | To discharge contaminants (namely odour and dust) to air from a composting operation. |
| RC-2024-0136-02 | Discharge Permit | To discharge contaminants to land in circumstances where they may enter water associated with a composting operation. |
| RC-2024-0136-03 | Discharge Permit | To discharge contaminants from composting leachate to land by irrigation to land in circumstances where they may enter water. |

LOCATION

1047 Kaniere-Kowhitirangi Road, Kokatahi – Lot 2 DP 1946 and Lot 2 DP 574750

MAP REFERENCE

At or about NZTM: E1439063 N5255590

## CONDITIONS

Pursuant to Section 108 of the Resource Management Act 1991, the Resource Consents include the following conditions:

General Conditions Applying to All Consents

- The discharges must be carried out in general accordance with the details contained in the consent application submitted to the West Coast Regional Council, except where inconsistent with these conditions.

- Any person working under this consent must have a copy of the consent conditions on site and present it to an officer of the West Coast Regional Council upon request.

- The Consent Holder must ensure that the following effects do not occur beyond the property boundary:

  - Any discharge of particulate matter or deposition of particulate matter on any land or structure is not objectionable beyond the property boundary; or

  - Any discharge of odour is not noxious, dangerous, offensive or objectionable beyond the property boundary.

Advice Note: The site is defined as the property boundary is legally identified as Lot 2 DP 1946 BLK IX Kaniere SD and Lot 2 DP 574750.

- The discharge beyond the property boundary must not result in non-compliance of the Resource Management (National Environmental Standards for Air Quality) Regulations 2004.

- The Consent Holder must record any complaints it receives in complaints register which details those complaints, onsite causes for the complaints, the weather conditions at the time of the complaint and the action(s) taken to address the complaint and prevent it from reoccurring.  A copy of the complaints register must be provided to the West Coast Regional Council within 15 working days of the complaint being received or upon request by the West Coast Regional Council.

- Pursuant to Section 128 of the Resource Management Act 1991, the West Coast Regional Council may review the conditions of this consent by serving notice within a one month period of each anniversary of the date of commencement of the consent for any of the following purposes:

  - To deal with any adverse effect on the environment which may arise from the exercise of this consent and which it is appropriate to deal with at a later stage; or

  - To require the Consent Holder to adopt the best practicable option to remove or reduce any adverse effect on the environment; or

  - To comply with the requirements of a Regional Plan.

Specific Conditions Applying to RC-2024-0136-01 (Discharge to air from composting operations including odour and dust)

- The discharge to air must not cause an odour which is offensive or objectionable beyond the boundary of the property on which this consent is exercised.

- The raw materials used in the manufacture of compost must be limited to the items listed in Table 1 below:

Table 1 – Feedstock

| Feedstock | Annual limit (tonnes) |
| --- | --- |
| Paunch Grass | 500 |
| Bark Chip / Sawdust / Old bailage and silage / Used herd home and calf bedding (wood chips & manure) | Cumulative total of 3000 |
| Abattoir DAF Wastewater Sludge (industrial) | 500 |
| Moss fines | 100 |

- The total amount of raw material received on the site to produce compost must not exceed 4,100 tonnes per year.

- Notwithstanding Conditions 8 and 9, before the onset of composting the Consent Holder must provide the Consent Authority for certification a register of approved feedstocks, specifying types, sources (i.e. named supplier) and characterisation data. New feedstocks or volume changes require a variation application to the consent conditions pursuant to s.127 of the Act that includes a risk assessment for odour, contaminants, and any compliance history.

- The Consent Holder must mandate a digital register through Excel with PDF backups (or equivalent) for all certified incoming materials, tracked via weighbridge dockets. The record must record tonnages, origins, and dates, and be available for Council audits.

File note to Condition 11: The annual reporting should include mass balances to track material lifecycles to align with broader waste minimisation goals under the Waste Minimisation Act 2008.

- The Consent Holder must ensure that where possible, compost operations (such as turning of windrows) is undertaken at times which are least likely to cause objectionable odour effects on neighbours, with consideration given to wind speed and direction.

- Prior to first exercise of this consent, the Consent Holder must install and undertake ongoing maintenance of an electronic data logging weather station by a suitably qualified and experienced person in an unobstructed central location on the site to record weather conditions on-site and for the purposes managing composting and composting process and assessing the need for mitigation measures against objectionable odour emissions and to inform the Compost Site Management Plan (CSMP) review.

Resource consent conditions during compost pasteurisation phase

- The ratio of C:N (carbon to nitrogen) within the composting organic mixture must be maintained within the range of 25:1 to 35:1 at the start of composting.

- The Consent Holder must ensure that the compost windrow temperature is greater than or equal to 55 degrees Celsius for a minimum of 15 consecutive days and that the windrows are turned at least 5 times during this period.

- The Consent Holder must ensure that the compost windrow oxygen concentration is always greater than 10%.

- The Consent Holder must ensure that the moisture content within the compost windrow is maintained between 45% and 60%.

- The Consent Holder must ensure that the pH of the compost windrow is maintained between 6.5 and 8.

- The Consent Holder must monitor and record the operational parameters referred to in Conditions 14 - 18. This information must be made available to the West Coast Regional Council upon request or when non-complying with conditions 14-18 with reasoning and remedies to comply with conditions which must include:

a. Daily visual and olfactory assessment of the condition of the compost windrows.

b. Daily monitoring of temperature using a sampling temperature probe with an accuracy of +/-0.5 degrees Celsius.

c. Daily monitoring of oxygen concentration at a depth of at least 200mm inside the windrow using an oxygen sampling probe with a minimum accuracy of +/- 1%.

d. Twice weekly monitoring of the moisture content as measured using Appendix F of New Zealand Standard 4454:2005 Composts, Soil Conditioners and Mulches.

e. Fortnightly monitoring of pH using Appendix A of New Zealand Standard 4454:2005 Composts, Soil Conditioners and Mulches.

Resource consent conditions during compost maturation phase

- During the composting maturation phase the Consent Holder must measure the temperature, moisture content, and oxygen level twice weekly. This information must be recorded and made available to the West Coast Regional Council upon request.

- The Consent Holder must maintain records of any odour or dust complaints received and provide these to the West Coast Regional Council on request.

- The complaint records required by Condition 21 of this resource consent must include:

a. Name and location/address and telephone number of complainant when odour/dust is detected (if provided);

b. Date and time of the odour/dust detection;

c. Weather conditions, including wind direction and speed at the composting facility when odour/dust detected;

d. Intensity, duration and offensiveness of the odour/dust complained of;

e. Any possible cause for the odour/dust complained of; and

f. Any corrective action taken.

Other matters

- Twenty working days prior to exercise of this consent, the Consent Holder must prepare and submit to the West Coast Regional Council, a Composting Site Management Plan (CSMP) for approval that it complies with the requirements of this consent. The composting operation must be managed in accordance with this plan, and a copy must be always held onsite by the operator.

The plan must include, but not be limited to, the following:

a. Describing measures to be undertaken to achieve compliance with the conditions of this consent;

b. A detailed description of the compost operation, including hours of operation;

c. Acceptable raw material, unacceptable raw material, site access and security; and

d. A description of activities or operations that may generate odour and/or dust, and measures in place to avoid, remedy or mitigate those discharges; and

e. A description of how the leachate and stormwater collection system will operate, including when and how collected leachate will be pumped onsite back over the windrows and/or off site for disposal to land; and

f. The means of receiving, recording in writing, and dealing with any complaints; and

g. A list of site management structure and responsibilities; and

h. A list of afterhours contact details in case of any emergency of problems; and

i. Contingency plans in the event of system malfunctions or breakdowns; and

j. Procedures for effective pasteurisation of compost in accordance with Appendix K of New Zealand Standard 4454:2005; and

k. Monitoring programme for airborne spores/pathogens and dust.

- The CSMP must be reviewed once every two years, or upon substantiated complaints or non-compliance, or as requested by the West Coast Regional Council, and updated as required, and the outcome of the review and any update, must be provided in writing to the West Coast Regional Council. At all times, the Consent Holder must ensure that the West Coast Regional Council has a copy of the most recent version of the CSMP.

Specific Conditions Applying to RC-2024-0136-02 and RC-2024-0136-03 (Discharge from composting and leachate irrigation)

- Upon the anniversary date (12 months) from the commencement of these consents, and at annual intervals thereafter, the Consent Holder must supply the West Coast Regional Council with the following information:

A detailed scale plan or aerial photograph showing:

- The locations of the areas receiving the compost leachate within the last 12 months together with the quantities discharged; and

- Locations of any natural watercourses and drains in proximity to compost leachate discharged.

- The Consent Holder must ensure that stormwater is diverted away from any Compost stockpile location(s) and any discharge from the Compost stockpile(s) are contained to ensure that neither the discharge nor any contaminants arising from the stockpile(s) are able to enter any surface water body, including a drain.

- The Consent Holder must not discharge compost leachate within:

- 20 metres of any river, creek or stream;

- 20 metre of any drain with flowing water;

- 50 metres of any well or bore used for potable water supply.

- The Consent Holder must ensure that

- There is no visible runoff of compost leachate into surface water bodies or drains;

- There is no ponding or visible surface flow of effluent, or pasture burning; and

- The cumulative application rate of compost leachate is at a rate not exceeding the equivalent of 200kg compost leachate total-nitrogen/ha/year and must not exceed 20mm in depth per application.

- For the purpose of compliance with condition 28(c), the consent holder must provide annual report to West Coast Regional Council including the calculations of total-N concentrations and compost leachate volumes applied on a hectare basis.

- The Consent Holder must provide a report to the West Coast Regional Council annually together with the information required under Condition 23 and the sampling under Condition 28, this report must include annual sampling in accordance with the New Zealand Standards for Composts, Soil Conditions and Mulches as contained in Table 3.1 for Physical, Chemical and Biological Requirements.

- Notwithstanding Condition 29, at monthly intervals for no less than a contiguous 12-month period from the date of discharging leachate from the composting activities and upon written request from the West Coast Regional Council thereafter, the Consent Holder must undertake sampling at the following sampling points, for the following sampling parameters and for the duration stipulated within the request (i.e. if required after 12 month of sampling):

Sampling Points – Detailed on separate plan

- Point A – at or about NZTM E1439310 N5255665 Duck Creek (U/S);

- Point B – at or about NZTM E1439181 N5255852 Duck Creek (D/S);

- Point C – at or about NZTM E1439140 N5255365 unnamed tributary (U/S);

- Point D – at or about NZTM E1439112 N5255853 unnamed tributary (D/S); and

- Point E – Compost leachate within the leachate holding/collection pond.

Sampling Parameters

- TBOD5;

- Total Suspended Solids;

- E. Coli (membrane filtration)

- Ammoniacal-Nitrogen;

- Nitrate-Nitrogen

- Total Nitrogen;

- Dissolved Reactive Phosphorus (DRP);

- Total Phosphorus.

Sampling Requirements

The sampling required under the request must only be undertaken.

On each separate occasion where sampling is carried out it must be undertaken as follows:

- A suitably qualified person must carry out sample collection;

- Analysis must be undertaken by an accredited laboratory;

- Photographs must be taken of the discharge and sampling points;

- Sampling and taking photographs must be at approximately the same time of day.

- Results from the sampling and the photographs required by Condition 30 must be forwarded to the West Coast Regional Council within 10 days of the results of each sampling occasion being received by the Consent Holder.  The time of day the samples were collected and the photographs taken must be included with the results.

- The discharge must not cause the natural (background) water quality of Point A or Point D, to fall below the following standards when measured at Point B or Point C respectively:

- The TBOD5 must not be increased by more than 20%;

- The concentration of ammoniacal-nitrogen must not be increased by more than 0.03 mg/L; and

- For Conditions 32(a) and (b) and Condition 33, sampling must not occur when there has been greater than 30 mm of rainfall in the previous 24 hours as measured at the Butchers Creek rain gauge.

- The concentration of E. Coli must not be increased to or greater than 130 E. coli/ 100ml at Point A or Point D when it is 130 E.coli/100ml or less at Point B or Point C respectively, or must not be increased by more than 130 E coli/100 ml at Point A or Point D when it is greater than 130 E. coli/100 ml at Point B or Point C respectively.

- Notwithstanding Conditions 32 and 33, the discharge of compost leachate must not result in the following effects within any receiving water body at or beyond Point A or Point D:

- The production of any conspicuous oil or grease films, scums or foams, or floatable

suspended materials;

- Any conspicuous change in the colour or visual clarity;

- Any emission of objectionable odour;

- The rendering of fresh water unsuitable for consumption by farm animals;

- Any significant adverse effects on aquatic life.

- The Consent Holder must provide the West Coast Regional Council with any further information, which the West Coast Regional Council may reasonably request after reviewing the results of its monitoring under Condition 7 within one month of the request being made or at a later date subject to the West Coast Regional Council’s written agreement.

Resource consent conditions for the receipt of Abattoir DAF Wastewater Sludge (industrial), or other odorous waste streams

- Following receipt of the abattoir DAF Wastewater Sludge or any other odorous material detailed in Condition 8 initial storage shall occur in a purpose built walled bunkers. The site shall contain a maximum of two walled bunkers with each bunker having a maximum volume of no more than 100 m³

- The walled bunkers shall be operated continuously whenever material is stored within the bunker to maintain a negative air pressure within the bunker. Air from the bunker shall be vented through a biofilter. The biofilter shall be designed, constructed, operated and maintained to ensure:

a. A design empty bed residence time of a minimum of 15 seconds;

b. A media depth of no less than 1m;

c. An even distribution of air flow through a uniform media bed;

d. An operating pH in the range 5 – 9;

e. A filter medium that is visibly damp below the surface and with a moisture content 40 - 60% on a dry weight basis; and

f. Shell or carbonaceous material must be incorporated within the biofilter bed.

- The static pressure drop across the biofilter will be measured and recorded monthly.

##### TERM

The term of this consent is 10 years from the date of commencement.

Attachment “Dr Joe Wildy – High level review of conditions.pdf”